Title: HCI & CG Lab 01 Assignment

Student Name: Abbas Khan

Student-id:  CSE/2K24/89

Submitted-Date: 11-Sept-2026

Task 1: Display PPI/DPI Calculator

Objective: To calculate total pixels, aspect ratio, display PPI/DPI, and display-density category.

- Code

import math

def get_positive_int(prompt):

while True:

try:

value = int(input(prompt))

if value > 0:

return value

print("Please enter a positive whole number.")

except ValueError:

print("Invalid input. Please enter a whole number.")

def get_positive_float(prompt):

while True:

try:

value = float(input(prompt))

if value > 0:

return value

print("Please enter a value greater than zero.")

except ValueError:

print("Invalid input. Please enter a numeric value.")

width_px = get_positive_int("Enter horizontal resolution (pixels): ")

height_px = get_positive_int("Enter vertical resolution (pixels): ")

diagonal_inches = get_positive_float("Enter physical diagonal size (inches): ")

total_pixels = width_px * height_px

gcd_value = math.gcd(width_px, height_px)

aspect_width = width_px // gcd_value

aspect_height = height_px // gcd_value

diagonal_pixels = math.sqrt(width_px ** 2 + height_px ** 2)

ppi = diagonal_pixels / diagonal_inches

if ppi < 100:

category = "Low Density (Standard Monitor)"

elif ppi <= 200:

category = "Medium Density (HD Display)"

else:

category = "High Density (Retina / Mobile)"

print("\n--- DISPLAY METRICS ANALYSIS ---")

print(f"Total Pixel Count : {total_pixels:,} pixels")

print(f"Aspect Ratio      : {aspect_width}:{aspect_height}")

print(f"Calculated DPI    : {ppi:.2f} DPI")

print(f"Density Category  : {category}")

- Desktop output Terminal

- Smartphone output Terminal

Task 2: Synthetic Image Matrix Creation

Objective: To create a 300 × 400 × 3 RGB image array using NumPy and fill its four quadrants with different colors.
- Code

import numpy as np

import matplotlib.pyplot as plt

height = 300

width = 400

channels = 3

image = np.zeros((height, width, channels), dtype=np.uint8)

mid_height = height // 2

mid_width = width // 2

# Top-left: Red

image[:mid_height, :mid_width] = [255, 0, 0]

# Top-right: Green

image[:mid_height, mid_width:] = [0, 255, 0]

# Bottom-left: Blue

image[mid_height:, :mid_width] = [0, 0, 255]

# Bottom-right: White

image[mid_height:, mid_width:] = [255, 255, 255]

print("--- SYNTHETIC MATRIX METRICS ---")

print(f"Array Shape (H, W, C) : {image.shape}")

print(f"Data Type             : {image.dtype}")

print(f"Total Elements        : {image.size:,} values")

print(f"Memory Footprint      : {image.nbytes:,} bytes ({image.nbytes / 1024:.2f} KB)")

plt.figure(figsize=(8, 6))

plt.imshow(image)

plt.title("Synthetic RGB Image: Four Color Quadrants")

plt.axis("off")

plt.tight_layout()

plt.savefig("task2_synthetic_image.png", dpi=200)

plt.show()

- Terminal Output

Task 3: Channel Slicing and Isolation

Objective: To extract red, green, and blue channels from sample.jpg and display their isolated views and grayscale intensity maps.
- Code

import os

import numpy as np

import matplotlib.pyplot as plt

from PIL import Image

script_folder = os.path.dirname(os.path.abspath(__file__))

image_path = os.path.join(script_folder, "sample.jpg")

if not os.path.exists(image_path):

print(f"Error: '{image_path}' was not found.")

print("Place sample.jpg in the same folder as this Python file.")

raise SystemExit

img = np.array(Image.open(image_path).convert("RGB"))

# Extract 2D color-channel intensity grids

red_channel = img[:, :, 0]

green_channel = img[:, :, 1]

blue_channel = img[:, :, 2]

# Create RGB arrays containing only one active channel

red_only = np.zeros_like(img)

green_only = np.zeros_like(img)

blue_only = np.zeros_like(img)

red_only[:, :, 0] = red_channel

green_only[:, :, 1] = green_channel

blue_only[:, :, 2] = blue_channel

print("--- CHANNEL EXTRACTION SUMMARY ---")

print(f"Original Image Shape     : {img.shape}")

print(

f"Red Channel 2D Shape     : {red_channel.shape} | "

f"Mean Intensity: {red_channel.mean():.2f}"

)

print(

f"Green Channel 2D Shape   : {green_channel.shape} | "

f"Mean Intensity: {green_channel.mean():.2f}"

)

print(

f"Blue Channel 2D Shape    : {blue_channel.shape} | "

f"Mean Intensity: {blue_channel.mean():.2f}"

)

fig, axes = plt.subplots(2, 3, figsize=(15, 9))

# Top row: individual color images

axes[0, 0].imshow(red_only)

axes[0, 0].set_title("Red-Only Image")

axes[0, 0].axis("off")

axes[0, 1].imshow(green_only)

axes[0, 1].set_title("Green-Only Image")

axes[0, 1].axis("off")

axes[0, 2].imshow(blue_only)

axes[0, 2].set_title("Blue-Only Image")

axes[0, 2].axis("off")

# Bottom row: grayscale channel intensity maps

axes[1, 0].imshow(red_channel, cmap="gray")

axes[1, 0].set_title("Red Channel Intensity")

axes[1, 0].axis("off")

axes[1, 1].imshow(green_channel, cmap="gray")

axes[1, 1].set_title("Green Channel Intensity")

axes[1, 1].axis("off")

axes[1, 2].imshow(blue_channel, cmap="gray")

axes[1, 2].set_title("Blue Channel Intensity")

axes[1, 2].axis("off")

plt.tight_layout()

plt.savefig("task3_channel_slicing.png", dpi=200)

plt.show()

-Terminal Output

- 2 × 3 output figure

Task 4: Downsampling and Pixelation

Objective: To reduce an image by taking every 8th pixel using NumPy array slicing and then re-expand it to observe pixelation.

- Code

import os

import numpy as np

import matplotlib.pyplot as plt

from PIL import Image

image_path = "sample.jpg"

N = 8

if not os.path.exists(image_path):

print(f"Error: '{image_path}' was not found.")

print("Place sample.jpg in the same folder as this Python file.")

raise SystemExit

img = np.array(Image.open(image_path).convert("RGB"))

original_height, original_width, channels = img.shape

# Take every N-th pixel from rows and columns

downsampled = img[::N, ::N, :]

# Expand pixels to create the blocky/pixelated image

expanded = np.repeat(np.repeat(downsampled, N, axis=0), N, axis=1)

# Crop in case dimensions are not perfectly divisible by N

expanded = expanded[:original_height, :original_width, :]

height_reduction = (1 - downsampled.shape[0] / original_height) * 100

width_reduction = (1 - downsampled.shape[1] / original_width) * 100

memory_savings = (1 - downsampled.nbytes / img.nbytes) * 100

print(f"--- DOWNSAMPLING ANALYSIS (N = {N}) ---")

print(f"Original Shape     : {img.shape} | Memory: {img.nbytes:,} bytes")

print(

f"Downsampled Shape  : {downsampled.shape} | "

f"Memory: {downsampled.nbytes:,} bytes"

)

print(f"Re-expanded Shape  : {expanded.shape} | Visual: Blocky Pixelation")

print(

f"Dimension Reduction: {height_reduction:.2f}% height reduction, "

f"{width_reduction:.2f}% width reduction"

)

print(f"Memory Savings     : {memory_savings:.2f}% data reduction")

fig, axes = plt.subplots(1, 3, figsize=(18, 6))

axes[0].imshow(img)

axes[0].set_title(f"Original Image\n{img.shape}")

axes[0].axis("off")

axes[1].imshow(downsampled)

axes[1].set_title(f"Downsampled Image (N={N})\n{downsampled.shape}")

axes[1].axis("off")

axes[2].imshow(expanded)

axes[2].set_title(f"Re-expanded / Pixelated\n{expanded.shape}")

axes[2].axis("off")

plt.tight_layout()

plt.savefig("task4_downsampling_pixelation.png", dpi=200)

plt.show()

- Terminal Output

- Output figure

Short Answer: RGB vs. RGBA Memory Consumption

RGBA uses 33.33% more memory than RGB because RGB stores three values per pixel: red, green, and blue. RGBA stores an additional fourth value, called the alpha channel, which controls transparency. When uint8 is used, each channel occupies 1 byte. Therefore, an RGB pixel uses 3 bytes, whereas an RGBA pixel uses 4 bytes. The extra 1 byte is one-third of the original 3-byte RGB storage, so RGBA requires 33.33% more memory.

MUHAMMAD ABBAS BABAR KHAN  •  2k24/CSE/89  •  ASSIGNMENT: HCI VS COMPUTER GRAPHICS